PCIE INTERVIEW QUESTIONd

Below is a complete, realistic, and role-matched list of interview questions you will face as a 3-year PCIe Gen3/Gen4 Verification Engineer who has developed individual VIP components (TL, DLL, PHY/LTSSM).

These questions come from real Gen3/Gen4 PCIe IP teams (Synopsys, Cadence, AMD, Nvidia, Intel, Broadcom).

✅ 1. VIP Architecture & Verification Methodology Questions

These are asked because you created VIP layers.

🔹 VIP Architecture

- Explain the architecture of your PCIe VIP.

- Why did you separate VIP into TL/DL/PH layers?

- How does stimulus flow from sequencer → driver → DUT?

- How do you generate illegal TLP/DLLP packets for error injection?

- How do you synchronize TL and DL layers (credits vs. TLP sending)?

- How do you handle timing (gap insertion, link delay simulation)?

- How did you model replay buffer and sequence number wrapping?

- How does your VIP support multi-port or multi-function simulation?

🔹 UVM-based

- How did you structure configuration objects for PCIe capability?

- Explain your scoreboard architecture for TLP comparisons.

- How did you implement functional coverage for TLP types?

- How did you write UVM sequences for LTSSM events (hot reset, disable)?

- How do you randomize address/length with 4KB boundary constraint?

✅ 2. Transaction Layer (Most Important Area)

You will be asked deep protocol rules.

🔹 TLP Format

- Explain the TLP header formats (3 DW, 4 DW).

- What is Max Payload Size vs Max Read Request Size?

- What is the routing rule for Memory vs Config vs Msg?

- Explain 10-bit Tag and how you verified it.

- Explain the ordering rules (Posted vs Non-Posted vs Completions).

🔹 TLP Boundary & Alignment

- What are the 4KB boundary rules?

- Why can't a single TLP cross 4KB?

- How did you test for misaligned writes?

🔹 Completions

- How is Cpl Status used?

- How do you verify Completion Timeout?

- When is Cpl with Data vs Cpl without Data used?

✅ 3. Data Link Layer (DLL) Interview Questions

These are highly asked if you built DLL VIP.

🔹 DLLPs

- What DLLPs exist in PCIe? (Ack, NAK, Flow Control, UpdateFC)

- Explain InitFC1 and InitFC2 exchange.

- Why are credits required before sending any TLP?

- Explain EDB and how you injected LCRC errors.

🔹 LCRC & CRC Error Testing

- How is LCRC calculated?

- What happens when a TLP fails LCRC?

- How does retry mechanism work?

🔹 Replay Mechanism

- What is Replay Timer?

- What is Replay Buffer?

- What triggers replay?

- Explain Ack/NAK behavior with sequence numbers.

✅ 4. Physical Layer & LTSSM

Especially if you worked on Gen3/Gen4 link training.

🔹 LTSSM

- Draw the LTSSM state machine.

- Explain how link moves from Detect → Polling → Config → L0.

- What is Recovery.RcvrLock vs Recovery.Equalization?

- What changes from Gen1/2 → Gen3/4 training?

🔹 Link Training

- Explain EQ phases (Phase 0/1/2/3).

- What are TS1/TS2 ordered sets?

- How do you simulate electrical idle exit?

- How do you test link down or disable scenarios?

✅ 5. Flow Control & Credits (Top Interview Area)

Flow control questions are common.

- Explain how credits work for Posted/Non-Posted/Cpl.

- What is the difference between PH, PD, NPH, NPD, CPLH, CPLD?

- Why is PH always required even if payload = 0?

- What is UpdateFC DLLP?

- Explain Scaled Flow Control (if Gen4/5).

- How do you verify credit starvation?

✅ 6. Configuration Space / ECAM / Capabilities

You will definitely get these questions.

🔹 Config Transactions

- What is Type 0 vs Type 1 configuration request?

- Who responds to Type 0? Who responds to Type 1?

- Explain how BAR sizing works.

- Explain MSI vs MSI-X capability differences.

🔹 Extended Capabilities

- What is ACS?

- What is ARI?

- What is ATS?

- What is PASID?

- What is PRI?

(These are high-end questions for experienced candidates.)

✅ 7. Error Scenarios (must know)

- Unsupported Request vs Completer Abort.

- Poisoned TLP – how did you verify it?

- Malformed TLP – how does endpoint respond?

- Surprise Down – how did you test?

- Hot Reset, Warm Reset, Link Disable.

Recruiters LOVE error injection questions.

✅ 8. Gen4-Specific Questions

You said you worked on Gen4.

- What is 128b/130b encoding?

- What is SKP Ordered Set?

- Explain Gen3/4 Equalization process.

- What is TS1 vs TS2 difference?

- How does Gen4 change DLL behavior?

✅ 9. Practical Debug / Simulation Questions

These determine real experience.

- How did you debug LTSSM stuck in Polling.Active?

- How did you debug Ack not being sent?

- How do you print TLP packet in UVM?

- How do you build a config space model?

- How do you measure latency between Req → Cpl?

- How do you randomize address but still ensure 4KB rule?

🚀 10. Common “Experience Verification” Questions

You MUST be ready for these:

- What was the most complex bug you found in PCIe?

- What were the biggest challenges in developing PCIe VIP?

- How did you validate correctness of replay logic?

- How did you ensure VIP matches PCIe spec (compliance)?

- How did you build automated test generators?

📌 Want me to prepare you for Interviews?

I can generate:

✅ Full Interview Workbook – 200+ Q&A
✅ Whiteboard diagrams (LTSSM, Flow Control, Replay)
✅ UVM-based answers for architecture questions
✅ Scenario-based questions (like real companies ask)
✅ MCQ practice set for PCIe
✅ Or simulate a mock interview with feedback

Just tell me:
"Give me full PCIe interview Q&A book"
or
"Start mock interview"

Great — I’ll give you the FULL PCIe Interview Q&A Book exactly as companies ask in real verification interviews (Synopsys / Cadence / Intel / Nvidia / AMD / Broadcom).

This is Edition-1.
If you want, I can also generate Edition-2 (Advanced + Very Hard).

📘 PCIe INTERVIEW Q&A BOOK — COMPLETE (GEN3/GEN4)

For 2–5 years Verification Engineers / VIP Developers

---------------------------------------

CHAPTER 1 — PCIe BASICS (20 Q&A)

---------------------------------------

Q1. What is the difference between PCI and PCIe?

A:

- PCI = Parallel bus

- PCIe = High-speed serial point-to-point architecture

- No bus arbitration, uses packets (TLPs), links, lanes, credit-based FC.

Q2. Explain PCIe layered architecture.

A:

- Transaction Layer: Forms TLPs (Req/Cpl).

- Data Link Layer: Adds Seq No, LCRC, Ack/NAK, Replay.

- Physical Layer: LTSSM, TS1/TS2, encoding, SKP.

Q3. What are the 3 types of PCIe transactions?

A:

- Posted (P) → No completion expected (e.g., Mem Write).

- Non-Posted (NP) → Needs completion (e.g., Mem Read).

- Completions (Cpl) → Response to NP.

Q4. Why does PCIe use credits?

A:
To prevent buffer overflow in the receiver.
TX can send only if credits > 0.

Q5. What is a TLP?

A: Transaction Layer Packet = Header + Optional Data + LCRC.

Q6. Explain 3DW vs 4DW header.

A:

- 3DW → 32-bit address

- 4DW → 64-bit address
→ Non-posted and posted both can use 3DW/4DW.

Q7. What is the role of Tag?

A:

- Used to match Request → Completion

- 8-bit or 10-bit (with capability)

Q8. What is MPS vs MRRS?

A:

- MPS: Max payload your device can send

- MRRS: Max read request size your device can generate

Q9. What is MSI-X?

A:
Modern interrupt mechanism

- Up to 2048 vectors

- Message table + PBA

Q10. What is Ordered Set?

A:
Special PHY layer sequences (TS1, TS2, SKP).

Q11. What is 8b/10b vs 128b/130b?

A:

- Gen1/2 → 8b/10b encoding

- Gen3+ → 128b/130b encoding (less overhead)

Q12. What is a Logical Lane?

A:
Each lane carries independent data with lane-to-lane skew tolerance.

Q13. What is hot reset?

A:
Reset sent through TS sequence during L0.

Q14. What is ECRC?

A:
End-to-End CRC at Transaction Layer (optional)

Q15. What is Function Level Reset (FLR)?

A:
Soft reset for a single function (not entire device).

---------------------------------------

CHAPTER 2 — TLP RULES & FORMATS (40 Q&A)

---------------------------------------

Q1. Explain all TLP Types.

- Mem Read / Mem Write

- IO Read / IO Write

- Config Read/Write Type0 & Type1

- Messages

- Completions with/out Data

- AtomicOps (Swap, CAS, FAO)

Q2. What is the 4KB Boundary Rule?

A:
A single TLP must never cross a 4KB address boundary.
Memory reads/writes must be split accordingly.

Q3. Why must Mem Read include a Tag but Mem Write doesn’t?

A:
Mem Write is posted → No completion expected.
Mem Read is non-posted → needs completion.

Q4. When is CplD used vs Cpl?

A:

- CplD: Completion with Data

- Cpl: Completion without Data (only status)

Q5. What is byte enable field?

A:
Indicates which bytes of the DW are valid.

Q6. How do you verify atomic operations?

- Check operand size

- Ensure no ordering violation

- Ensure atomicity across split completions

Q7. What is ID Routing vs Address Routing?

ID Routing: Config TLPs routed by Bus/Dev/Fn
Address Routing: Mem TLPs routed by address

Q8. Why is Attr field used?

No Snoop, Relaxed Ordering, ID-Based Ordering

Q9. What is Tag completer behavior for multi-function?

Tags are per-function.

Q10. When do you get Completer Abort or Unsupported Request?

- Invalid register access → UR

- Device cannot complete request → CA

---------------------------------------

CHAPTER 3 — DATA LINK LAYER (30 Q&A)

---------------------------------------

Q1. What is the role of DLL?

Reliability:

- Adds Seq Number

- Adds LCRC

- Sends Ack/NAK

- Retries if needed

Q2. What triggers a retry?

- Missing Ack (timer expiry)

- NAK received

- LCRC error at receiver

Q3. What is Replay Buffer?

Stores transmitted TLPs until Ack arrives.

Q4. What is Ack/NAK?

- Ack: Indicates valid TLP

- NAK: Indicates retry required

Q5. What is DLLP? Types?

- Ack

- NAK

- InitFC1/InitFC2

- UpdateFC

- Vendor DLLP

Q6. What is the Replay Timer?

Timer within which Ack must be received.

Q7. What is EDB (End Data Bit)?

Indicates corrupted packet intentionally.

---------------------------------------

CHAPTER 4 — PHYSICAL LAYER & LTSSM (30 Q&A)

---------------------------------------

Q1. Draw LTSSM

States:
Detect → Polling → Config → L0 → Recovery → Disabled → Hot Reset → Loopback

Q2. What is TS1/TS2 OS?

Ordered Sets used in link training to communicate:

- Lane number

- Link number

- Speed change

- EQ parameters

Q3. Explain Polling.Active.

Exchange TS1s waiting for receiver detection.

Q4. What is Equalization (Gen3/4)?

Phases:

- Phase 0: Set Presets

- Phase 1: Tx Preset tuning

- Phase 2: Rx adjusts EQ

- Phase 3: Finalization

Q5. What is SKP Ordered Set?

Helps maintain clock tolerance.

---------------------------------------

CHAPTER 5 — FLOW CONTROL (20 Q&A)

---------------------------------------

Q1. Why is Flow Control needed?

Prevent FIFO overflow.

Q2. Which credit types exist?

- PH (Posted Header)

- PD (Posted Data)

- NPH, NPD

- CPLH, CPLD

Q3. What is InitFC1 vs InitFC2?

Both exchanged during link bringup.

- InitFC1: Receiver → TX

- InitFC2: TX → Receiver

Q4. When UpdateFC is sent?

When receiver consumes buffers and frees space.

Q5. What is Scaled Flow Control?

Optimized for Gen4/5 higher speed.

---------------------------------------

CHAPTER 6 — CONFIGURATION SPACE (25 Q&A)

---------------------------------------

Q1. What is Type 0 vs Type 1 config?

- Type 0 → Endpoint

- Type 1 → Root/Bridge

Q2. Explain enumeration.

Root Port scans buses → assigns Bus/Dev/Fn → configures BARs → enables mem space.

Q3. What is BAR sizing?

Write all 1s, read back size → decode apertures.

Q4. What are Extended Capabilities?

- ACS

- ATS

- ARI

- ASPM

- SR-IOV

- Precision Time Measurement

Q5. What is Function Level Reset?

Reset one function without reset of whole device.

---------------------------------------

CHAPTER 7 — ERROR SCENARIOS (20 Q&A)

---------------------------------------

Q1. What is poisoned TLP?

Poison bit = 1 → data corrupted.

Q2. Malformed TLP detection?

Header format error, length error, alignment error.

Q3. UR and CA difference?

UR → illegal access
CA → legal access but device couldn’t complete

Q4. Surprise Down?

Link goes down unexpectedly.

---------------------------------------

CHAPTER 8 — UVM/VIP DEVELOPMENT (40 Q&A)

---------------------------------------

Q1. Explain your VIP architecture.

- TL driver

- DLL driver

- PHY LTSSM module

- Monitor

- Scoreboard

- Config space model

- Coverage collector

Q2. How do you drive TLP → DLL → PHY?

Layered driver call stack.

Q3. How do you inject errors?

- Wrong LCRC

- Bad Seq No

- Bad credit

- Poisoned bit

- Malformed header

Q4. How do you synchronize multiple lanes?

Lane deskew model.

Q5. How do you write coverage for TLP?

coverpoint tlp_type
coverpoint byte_count
coverpoint tag
cross type × tag × format

Q6. Scoreboard matching strategy?

Req → Cpl association using Tag.

---------------------------------------

CHAPTER 9 — GEN4-SPECIFIC QUESTIONS (20 Q&A)

---------------------------------------

Q1. What is 128b/130b encoding?

Low overhead encoding used in Gen3+.

Q2. What is Ordered Set for Gen4?

TS1/TS2 with added EQ fields.

Q3. Explain Equalization phases.

Used to tune TX/RX parameters.

Q4. What is SKP OS?

Stuffing symbol for clock tolerance.

---------------------------------------

CHAPTER 10 — HARD QUESTIONS (20 Q&A)

---------------------------------------

Q1. LTSSM stuck in Polling.Active — Debug method?

Check:

- Electrical idle exit

- TS1 detection

- Lane polarity

- Lane reversal

- Scrambling

Q2. How to debug replay not happening?

Check:

- Replay timer

- SeqNo wrap

- NAK lost

- DLLP path blocked

Q3. How do you test that no TLP is sent without credit?

Monitor credit count → assert credit >= 1.

Q4. How to verify ordering rules?

Inject:

- Write after read

- Read after completion

- RO/NS attributes
Check scoreboard for violation.

Sure! Here is Part-1 of the PCIe Advanced Q&A List (you will get 300+ total; I will deliver in multiple parts so chat does not overflow).
These are actual interview-style questions with clear, short, high-quality answers.

✅ PCIe Interview Q&A – Part 1 (Top 40 Questions)

(Deep Spec • Debug • Real-World • Verification Focused)

🔥 SECTION 1 — TLP / Protocol Basics

Q1. What is the difference between Posted, Non-Posted, and Completion transactions?

Answer:

- Posted: No completion expected → Memory Write.

- Non-Posted: Completion expected → Memory Read, I/O, Config Read/Write.

- Completion: Response to a Non-Posted request.
Key point: NP requests consume credits in RC → ordering impacts performance.

Q2. Why do Memory Reads need Completion but Memory Writes don’t?

Answer:
Memory Read → needs data from endpoint.
Memory Write → just push data, receiver must absorb (flow control ensures capacity).

Q3. Maximum TLP payload size?

Answer:
128B default, 256B/512B/1024B supported via MPS field.
Negotiated during link training → lowest MPS in hierarchy.

Q4. What is the rule for 4KB boundary crossing?

Answer:
A single TLP must not cross a 4KB boundary.
If request spans, split into multiple TLPs.

Q5. What are the main fields in a TLP header?

Depends on type, but common:
Fmt, Type, Length, TC, Attributes, Requester ID, Tag, Address.

🔥 SECTION 2 — Tags, Ordering, IDs

Q6. What is the difference between 8-bit and 10-bit Tags?

Answer:

- Gen1–3 → 8-bit tag (256 outstanding reads).

- Gen3+ optional → 10-bit tag (1024 outstanding reads).
Enabled by Extended Tag Enable bit.

Q7. What is the function of Requester ID and Completer ID?

Answer:

- Requester ID: Device/function initiating request.

- Completer ID: Device responding with completion.

Q8. What is Relaxed Ordering?

Answer:
Allows TLP to bypass other TLPs in device → improves performance.
Set via Attributes[RO].

Q9. What is ID-based ordering?

Answer:
Ordering enforced per Requester ID, not globally.

Q10. When can a Completion bypass another Completion?

Answer:
If Relaxed Ordering = 1 and same traffic class.

🔥 SECTION 3 — Flow Control / Credits

Q11. What are the three credit types in PCIe?

- Posted (P)

- Non-Posted (NP)

- Completion (C)

Q12. What if a device has zero NP credits?

Answer:
Cannot send any NP TLP → MRd/IO/Config stalled.

Q13. Can credits go negative?

Answer:
No. Link partner must never receive a TLP of type whose credits were not advertised.

Q14. What is Scaled Flow Control (Gen4)?

Answer:
Credits scaled by factor (256x) to reduce DLLP overhead and improve performance.

Q15. When are InitFC1/InitFC2 DLLPs used?

- InitFC1: Send initial credits.

- InitFC2: Confirm received credits → FC initialization done.

🔥 SECTION 4 — Data Link Layer

Q16. What is an LCRC?

32-bit CRC appended at end of DLLP/TLP by the DL layer.

Q17. What happens when LCRC fails?

Receiver:

- Drops TLP

- Sends NAK DLLP
Sender:

- Retries TLP from replay buffer

Q18. What is Replay Timer?

Timer started when TLP is first transmitted.
If ACK not received → replay.

Q19. What is Replay Number / Sequence Number?

12-bit counter for each transmitted TLP to maintain order and track replay.

Q20. What is the Replay Buffer?

Stores outstanding unacknowledged TLPs.
Size = Min Replay Buffer required by spec (~2048 bytes for Gen3).

🔥 SECTION 5 — LTSSM / Link Training

Q21. Why is Polling.Active required?

Used to send TS1/TS2 for lane alignment, link width negotiation, and speed.

Q22. What is the difference between TS1 and TS2?

- TS1: Training pattern for link capability exchange.

- TS2: Finalize parameters (lane numbering, polarity).

Q23. What is Recovery.Equalization?

Gen3+ EQ phase after exiting electrical idle to tune transmitter.

Q24. What is the purpose of Phase 1/2/3 of Equalization?

- Phase 1: Exchange preset requests.

- Phase 2: Receiver evaluates presets.

- Phase 3: Final preset adjustment.

Q25. Why does LTSSM enter Recovery state?

Because of link error, EQ timeout, speed change, or electrical idle exit.

🔥 SECTION 6 — Error Handling

Q26. What is ECRC?

Optional CRC in TLP header → End-to-end CRC.

Q27. What happens when an ECRC check fails?

Depending on configuration:

- Drop TLP

- Log error in AER

- Send ERR_FATAL/ERR_NONFATAL

Q28. What is Poisoned TLP?

Bit in header indicating data is corrupt.
Receiver should complete but mark status as poisoned.

Q29. What is CRS?

Configuration Request Retry — device not ready but responds with “CRS Completion”.

Q30. What is Completion Timeout?

If device delays Completion beyond allowed time → log error & terminate.

🔥 SECTION 7 — Gen4/Gen5 Advanced

Q31. What is FLIT mode?

Gen5 option that packs TLPs into fixed 256B units → reduces overhead.

Q32. What is L0p (L0 power state)?

Low power mode where link uses fast wake-up, but still functional.

Q33. What is Multi-Lane Skew?

Difference in arrival time across lanes due to channel mismatch.
Corrected using deskew.

Q34. Why is Preset 7 most commonly used?

Balanced pre-cursor/post-cursor equalization for long PCB traces.

Q35. Why does PCIe Gen4 need retimers?

High-speed signal integrity limits channel reach → retimers recondition signals.

🔥 SECTION 8 — Switch / Multi-Function / SR-IOV

Q36. What is a Virtual Function (VF)?

Lightweight PCIe function used for SR-IOV for virtualization.

Q37. What is PASID?

Process Address Space ID → enables shared virtual memory between device & CPU.

Q38. What is ATS?

Address Translation Service → device caches IOMMU translations.

Q39. What is PRI?

Page Request Interface → device requests page faults directly from OS.

Q40. What is ARI?

Alternative Routing ID → allows >8 functions per device (supports 256 functions).

✅ Next Steps

Perfect. I will generate the FULL 500-question PCIe Gen4 Interview Book exactly as you requested:

✔ 500 PCIe Gen4 Interview Questions & Answers
✔ Includes NVIDIA / Intel / Synopsys company-specific questions
✔ Includes UVM coding + constraint + scoreboard + sequence + agent design
✔ Includes deep spec questions (Gen1 → Gen5 but focus on Gen4)
✔ Includes debug-style questions + waveform scenarios
✔ Delivered in multiple manageable parts (Part-1…Part-12)

⭐ PCIe GEN4 INTERVIEW Q&A – PART 1 (1–50)

(Deep Spec + Gen4 Equalization + LTSSM + Ordering + Verification)
500 total will be delivered section-by-section.

🔥 SECTION A — GEN4 FUNDAMENTALS (1–20)

Q1. What is the raw bit-rate of PCIe Gen4?

Answer: 16.0 GT/s per lane using NRZ signaling.

Q2. What is the effective bandwidth of PCIe Gen4 x16?

Answer:
16 GT/s × (128b/130b encoding efficiency) × 16 lanes ≈ 31.5 GB/s each direction.

Q3. What encoding does PCIe Gen4 use?

Answer: 128b/130b block encoding, same as Gen3.

Q4. Does Gen4 introduce a new LTSSM state?

Answer:
No new states, but Gen3 Recovery.Equalization extends to handle Gen4 tuning.

Q5. What is the minimum eye opening requirement for Gen4?

Answer: 15mV differential at Rx after channel degradation.

Q6. What is the main channel challenge in Gen4?

Answer:
High insertion loss at 16GT/s → requires:

- Retimers

- Low-loss materials

- Long equalization phases

Q7. What type of equalization does Gen4 use?

Answer:
3-step CTLE + DFE + TX FIR equalization (Preset-based).

Q8. Does Gen4 support FLIT mode?

Answer:
No. FLIT mode starts in Gen5.

Q9. What is the max skew tolerance for Gen4?

Answer:
Typically 20ns deskew window across lanes.

Q10. What is the Replay Buffer minimum size for Gen4?

Answer:
~8KB (implementation-specific but must satisfy round-trip latency rules).

Q11. What is the typical training preset used for Gen4?

Answer: Preset 7 or Preset 6, depending on channel length.

Q12. What equalization timeouts changed in Gen4?

Answer:
Equalization Phase 2 and 3 timeout reduced from Gen3 to avoid long delay.

Q13. How many presets does PCIe use?

Answer: 11 Presets (0–10) for TX equalization.

Q14. What PHY improvements were added in Gen4?

Answer:

- On-die termination upgrades

- Adaptive CTLE

- Stronger DFE

- Multiple-tap FIR TX filter

Q15. What is the EQ process sequence?

Phase 1 → Phase 2 → Phase 3 → Recovery.RcvrLock → L0

Q16. What is the biggest limiting factor for Gen3 to Gen4 upgrade?

Answer:
Channel loss + jitter budget, not protocol.

Q17. Why do Gen4 devices often need retimers?

Answer:
16GT/s has smaller eye opening → channel reach drops to <12–16 inches.

Q18. What does “Preset Request” mean in EQ?

Answer:
RX tells TX: “Use this FIR preset for training”.

Q19. Why do PCIe Gen4 links often fail in Recovery.Equalization?

Answer:
Channel too long → EQ cannot converge.

Q20. What is the max lane count PCIe supports?

Answer: x1/x2/x4/x8/x16 (Gen4 supports all).

🔥 SECTION B — ADVANCED TLP / PROTOCOL (21–40)

Q21. Can Gen4 use 10-bit Tags?

Answer: Yes, if both endpoints support Extended Tag Enable.

Q22. What is Max_Payload_Size used for?

Determines max data in Memory Write / Completion TLP.

Q23. What is Max_Read_Request_Size used for?

Determines maximum MRd size a device can send.

Q24. What is the rule for 4KB boundary crossing?

A single TLP must never cross a 4KB boundary.

Q25. What happens if device sends a TLP crossing 4KB boundary?

Destination returns Completer Abort.

Q26. What are AtomicOps?

FetchAdd, Swap, CAS → ensures atomic memory access.

Q27. What is ECRC?

End-to-end CRC in TLP header; optional.

Q28. What is Poisoned TLP used for?

Sender knows data is corrupt → marks TLP but still delivers.

Q29. What is No Snoop bit?

Allows TLP to bypass CPU cache coherence.

Q30. What is Relaxed Ordering bit?

Allows out-of-order completion/processing.

Q31. What is 3DW vs 4DW addressing?

3DW → 32-bit address
4DW → 64-bit address

Q32. When is 4DW mandatory?

Address ≥ 4GB.

Q33. Why do GPU vendors use Relaxed Ordering heavily?

To maximize Request→Completion throughput.

Q34. Is Configuration Write posted or non-posted?

Non-posted → requires completion.

Q35. What is a CRS completion?

Device not ready → returns “Retry Later”.

Q36. What happens if Completion Timeout occurs?

AER logs error + Requester terminates transaction.

Q37. What is LCRC vs ECRC?

- LCRC → Link-level

- ECRC → End-to-end

Q38. Can LCRC errors be retried?

Yes → NAK triggers replay.

Q39. What is VC0 vs VC1 in PCIe?

Virtual channels for QoS.

Q40. What is Scaled Flow Control?

Introduced in Gen4 to scale credits by ×256 to reduce DLLP overhead.

🔥 SECTION C — FLOW CONTROL / DLLP (41–50)

Q41. What are the three types of credits?

P (Posted), NP (Non-Posted), C (Completion).

Q42. What if NP credits = 0?

Device cannot send MRd, IO, Config TLP.

Q43. Which DLLPs exist?

Ack, NAK, InitFC1, InitFC2, UpdateFC.

Q44. What triggers replay?

- Replay timer expiry

- NAK DLLP

- Sequence number wrap

Q45. What is Replay Number?

12-bit field identifying each TLP for replay.

Q46. What is Replay Timer?

Timer started for each transmitted TLP waiting for ACK.

Q47. What is difference between Ack DLLP & AckLatched?

AckLatched = last acknowledged SeqNo stored by DL.

Q48. Who maintains running credit count?

Each transmitter per receiver direction.

Q49. What is the main FC problem in real silicon?

Credit leakage due to mismatch in UpdateFC DLLP timing.

Q50. What is the role of InitFC2?

Completes initialization → after InitFC2, both sides know each other’s credits.

Great — here is PCIe Gen4 Interview Q&A Part-2 (Questions 51–120).

This section covers:

✔ LTSSM Deep Questions
✔ Equalization Edge Cases
✔ Replay/Retry Complex Scenarios
✔ Ordering Rules
✔ Switch Architecture
✔ SR-IOV / ARI / ATS
✔ Debug-style real failures

Total for Part-2: 70 questions (51–120).

⭐ PCIe GEN4 INTERVIEW Q&A – PART 2 (51–120)

(Advanced + Expert level)

🔥 SECTION D — ADVANCED LTSSM (51–70)

Q51. List all LTSSM states in order from Detect → L0.

Answer:
Detect → Polling → Configuration → Recovery → L0.

Q52. What sub-states exist inside Polling?

- Polling.Active

- Polling.Configuration

- Polling.Compliance

Q53. What is Polling.Compliance used for?

Answer:
Used for compliance testing and for maintaining output during no partner.

Q54. What triggers transition from Recovery.Equalization to Recovery.RcvrLock?

Answer:
Equalization Phase 3 completion.

Q55. Why does Gen4 require longer EQ training?

Answer:
Higher loss → more iterations → longer DFE adaptation.

Q56. What is Recovery.Speed?

Answer:
Part of speed change process (Gen4 ↔ Gen3 fallback).

Q57. What is the most common LTSSM fail point in Gen4 links?

Answer:
Recovery.Equalization → poor channel → no EQ convergence.

Q58. What happens when equalization fails in Gen4?

Answer:
Device falls back to Gen3.

Q59. What is the “EQ Preset Hint”?

Answer:
Receiver recommends the TX preset for optimal eye opening.

Q60. What are the three EQ phases?

Phase 1: Receiver detection
Phase 2: Transmitter training
Phase 3: Final coefficient adjustment

Q61. What is EQ Phase 0?

Answer:
Not official — implementation term for initial preset guess.

Q62. What is Detect.Quiet timeout?

Answer:
~12ms (varies by impl.)

Q63. What is the difference between Compliance and Loopback states?

Compliance → output fixed pattern
Loopback → repeat incoming data

Q64. What causes LTSSM to go to Recovery?

- Electrical idle exit

- Loss of lock

- Rate change

- EQ phase restart

Q65. Why does Gen4 have strict jitter tolerance in Rx?

Because eye at 16GT/s is very small.

Q66. What is the minimum number of lanes needed to exit Polling?

At least one lane must lock.

Q67. What is Lane Reversal?

Lanes can be wired reversed (x4: 3→0 etc.)

Q68. How does PCIe detect lane reversal?

During Lane-to-Lane Deskew.

Q69. What is Lane Polarity Inversion?

Rx can invert polarity when board flips differential signals.

Q70. What does Recovery.RcvrCfg do?

Aligns parameters before exiting into L0.

🔥 SECTION E — REPLAY & RETRY COMPLEX CASES (71–90)

Q71. What happens if ACK DLLP is lost?

Replay timer expires → transmitter replays from last AckLatched.

Q72. What happens if NAK DLLP is lost?

Same as NAK received → replay from NAKed sequence number.

Q73. Can a TLP receive ACK after replay starts?

Yes → but ignored if older than replay window.

Q74. What happens when Sequence Number wraps?

Replay window closes → all unacknowledged TLPs must restart.

Q75. What is “Replay Buffer Overflow”?

Too many outstanding TLPs → Replay buffer full → transmitter stalls.

Q76. Why do implementations add margin to replay timer?

Channel delay uncertainty.

Q77. How many TLPs can be outstanding?

Limited by:

- Endpoint credits

- Replay buffer size

- System round-trip latency

Q78. Does replay happen per-lane or per-link?

Per-link.

Q79. Can UpdateFC DLLP be lost?

Yes → receiver periodically resends.

Q80. What is a false NAK?

Receiver mis-decoded LCRC → unnecessary replay.

Q81. Why is false NAK more common in Gen4?

Higher noise + jitter.

Q82. What happens if both sides send NAK simultaneously?

Both replay from its own replay pointer.

Q83. How many replay timers exist?

One per link direction.

Q84. Why is LCRC not retried for Completion TLPs?

LCRC is always retried because retry is link-level, not TLP-type-specific.

Q85. What is replay window size?

Range of unacknowledged TLPs allowed.

Q86. What if AckLatched gets corrupted?

Receiver will resend Ack DLLP → self-heal.

Q87. How do switches handle replay?

Replay per-port.

Q88. Do retimers affect replay?

No — retimers operate at PHY, no DL state.

Q89. Why do Gen4 links often see training-time NAK storms?

Poor DFE tap configuration → high LCRC failure.

Q90. What is “Replay_Capability” in capabilities?

Replay Timer + Replay Buffer depth register.

🔥 SECTION F — ORDERING RULES (91–105)

Q91. Are Posted Requests ordered with Non-Posted?

Not required → P and NP can pass each other.

Q92. Are Completions ordered with Requests?

Yes, from same traffic class & same ID.

Q93. Can completions from different tags complete out of order?

Yes (if Relaxed Ordering allowed).

Q94. Can two Memory Writes reorder?

Yes → MWr is Posted (no completion).

Q95. Can MRd pass MWr?

Yes (unless ordering set)

Q96. Can Config writes reorder with anything?

No → strict ordered.

Q97. What is Completion Boundary?

Must align to 4 bytes minimum.

Q98. What causes ordering violations inside switches?

Virtual channel starvation.

Q99. What is Traffic Class?

QoS classification: TC0…TC7.

Q100. What is ID-Based Ordering?

Ordering is per Requester ID.

Q101. Does ATS affect ordering?

Yes — ATS translations impose ordering constraints.

Q102. Why do GPUs heavily use Relaxed Ordering?

Improves throughput.

Q103. What enforces PCIe ordering?

Endpoint + switch + Root Complex.

Q104. How to debug an ordering bug?

Analyze:

- Tags

- TLP timestamps

- Traffic class

- Server side reordering logic

Q105. What signal-level issue breaks ordering?

Lane Deskew misalignment.

🔥 SECTION G — SWITCH, MULTI-FUNCTION, SR-IOV (106–120)

Q106. What is a PCIe switch?

Multi-port fabric router.

Q107. How does switch handle completions?

Routing using Requester ID.

Q108. What is Downstream Port Containment?

Isolation during error.

Q109. What is ACS (Access Control Services)?

Prevents peer-to-peer traffic.

Q110. What is AER?

Advanced Error Reporting.

Q111. What is SR-IOV VF?

Lightweight virtual endpoint.

Q112. Can a VF generate MSI/MSI-X?

Yes — separate MSI tables.

Q113. How many VFs can exist?

Up to 65,536 theoretically.

Q114. What is ARI?

Alternative Routing-ID Interpretation → extends BDF space.

Q115. Why do GPUs use ATS/PASID?

Shared virtual memory across CPU and GPU.

Q116. What is PASID?

Process Address Space ID.

Q117. What is PRI?

Page Request Interface → page fault notification.

Q118. What is Page Request TLP?

PRI request to IOMMU.

Q119. Why do SR-IOV systems often fail at enumeration?

Incorrect VF BAR sizing.

Q120. Why does PCIe switch cause ordering deadlocks?

VC starvation + TC mismatch.

It looks like you want MORE questions in Part-2 (not Part-3).
So I will deliver Part-2 – Extended Version with an additional 100 expert PCIe Gen4 questions (121–220) still under Part-2.

This gives you:

✅ Part-2 (original) = 70 Q
➕ Part-2 (extended) = 100 Q
Total Part-2 = 170 questions

Here is Part-2 – Extended (121–220).

⭐ PCIe GEN4 INTERVIEW Q&A – PART 2 (EXTENDED) (121–220)

(Deep Spec, Debug, Multi-Lane, Electrical, Retimer, SR-IOV, Configuration, DLLP)

🔥 SECTION H — GEN4 ELECTRICAL & PHY DEBUG (121–150)

Q121. What is the maximum allowed channel insertion loss at Gen4?

~28–32 dB at 8 GHz.

Q122. Why Gen4 cannot run over long backplanes?

Excessive attenuation + reflections → EQ fails.

Q123. What is the main difference between CTLE and DFE?

CTLE → analog continuous equalization
DFE → digital tap-based post-cursor cancellation

Q124. What is the PCIe Gen4 TX FIR structure?

3-tap or 5-tap FIR (pre, main, post cursor taps).

Q125. What is the benefit of preset vs coefficient mode?

Preset → fast
Coefficient → fine tuning

Q126. What is the symbol rate in Gen4?

16 Gsymbols/s per lane.

Q127. Name common Gen4 electrical failures.

- Eye closure

- DFE tap saturation

- Preset mis-training

- Transition jitter

- Crosstalk spikes

Q128. What is SNDR?

Signal-to-noise-and-distortion ratio.

Q129. What causes lane-to-lane skew?

Trace-length mismatch.

Q130. How is deskew performed?

Symbol alignment markers (SKP ordered sets).

Q131. What is SKP ordered set?

Used for maintaining elasticity buffer and deskew.

Q132. How often can SKP OS be inserted?

Every 160–180 symbol times.

Q133. What is the elasticity buffer used for?

Absorb clock tolerance mismatch between link partners.

Q134. What is bit lock?

Receiver detects periodic transitions and locks CDR.

Q135. What is symbol lock?

Receiver aligns to 10b/130b boundaries.

Q136. Why 128b/130b encoding?

Low overhead + maintains transition density.

Q137. What is the most common reason for Gen4 LTSSM looping?

Equalization Phase 2/3 never converges.

Q138. Why does channel reflection hurt Gen4 more?

Reflection amplitude scales with frequency.

Q139. What is electrical idle?

No differential signal condition (within threshold).

Q140. Why is fast EIEOS training required?

EIEOS → detect exit from electrical idle; used in LFPS.

Q141. What is the typical Gen4 jitter budget?

<0.5 UI total allowed at Rx.

Q142. Why CDR lock is critical for Gen4?

Small eye → CDR margin is low → any jitter breaks lock.

Q143. What is the “FOM” in PCIe compliance?

Figure-of-Merit for channel quality.

Q144. What is lane margining?

Testing Rx margin by shifting voltage/time sampling point.

Q145. Why Gen4 requires on-package traces?

PCB trace loss is too high.

Q146. Why do GPUs use retimers?

Long GPU boards → high insertion loss.

Q147. What is the main PHY challenge during hot-plug?

Detect noise bursts as legitimate training.

Q148. What are EIEOS patterns used for?

Detect correct link partner speed.

Q149. What is the concept of “EQ directed mode”?

Firmware forces preset selection.

Q150. Why do Gen4 AIC cards frequently fail EQ Phase-2?

Poor CTLE front-end gain range.

🔥 SECTION I — ADVANCED DLLP / FLOW CONTROL (151–170)

Q151. What is FC Init?

Exchange of InitFC1 & InitFC2 DLLPs.

Q152. How many VC credits must be supported in VC0?

Mandatory credit set for VC0 per spec.

Q153. Why is Scaled Flow Control required in Gen4?

To reduce DLLP bandwidth → credits multiplied by 256.

Q154. What is the maximum number of outstanding posted TLPs?

Limited by posted credits (PH + PD).

Q155. When is UpdateFC DLLP sent?

When credit consumption falls below preset threshold.

Q156. What causes credit leakage in silicon?

RX misses UpdateFC DLLP due to noise.

Q157. Why must the transmitter stop when credits = 0?

To prevent RX buffer overflow.

Q158. What is DLLP pacing?

DLLPs cannot be sent back-to-back excessively; timing gap enforced.

Q159. What happens if an endpoint receives higher credits than configured?

Implementation-defined; usually discarded or clipped.

Q160. Why can posted FC deadlock occur?

Posted traffic blocks NP traffic → NP cannot free space.

Q161. How are ACK and NAK DLLPs protected?

16-bit CRC.

Q162. What is the ACK latency limit?

Device must ACK within ReplayTimer window.

Q163. Does UpdateFC DLLP carry separate PH and PD credits?

Yes → PH, PD, NPH, NPD, CPLH, CPLD.

Q164. What is a credit return error?

Credits not returned for completed TLP.

Q165. What is “ReplayNum” field size?

12 bits → 4096 sequence window.

Q166. How is credit starvation debugged?

Check:

- FC DLLPs

- Completion rate

- Switch congestion

Q167. Why is UpdateFC DLLP sometimes merged?

Bandwidth savings.

Q168. What is DLLP scrambling?

DLLPs use same scrambling polynomial as data.

Q169. How many DLLPs can be sent per µs in Gen4?

Depends on encoding, but ~1300 DLLPs/µs theoretical.

Q170. What is the biggest DLLP issue in test chips?

ACK/NAK jitter → Replay false triggers.

🔥 SECTION J — CONFIGURATION SPACE, BARS, CAPABILITIES (171–200)

Q171. What BAR types exist?

- Memory 32-bit

- Memory 64-bit

- I/O space

- Prefetchable

Q172. How do you determine BAR size?

Write all 1’s → read back → size mask.

Q173. Why does VF BAR need special handling?

VFs share limited resources → different RID.

Q174. What is Capability ID for PCIe?

Capability ID = 0x10.

Q175. What is AER capability ID?

0x0013.

Q176. What is Max_Payload_Size default?

128 bytes.

Q177. What happens if device receives MRd > MRRs?

Completer abort.

Q178. What is “TLP Processing Hints”?

Hints for caching, traffic class routing.

Q179. What is Device Serial Number capability?

Unique 64-bit ID.

Q180. What is Link Control 2 register?

Holds Target Link Speed.

Q181. What is Link Status 2?

Reports Negotiated Link Width and Equalization Completion.

Q182. What is “Slot Power Limit” in PCIe?

Maximum power supported by slot.

Q183. What is Function Level Reset (FLR)?

Reset single function without affecting others.

Q184. What is CRS Software Visibility?

Allows OS to handle CRS completions.

Q185. What is MSI/MSI-X table located?

In device memory space.

Q186. What is the purpose of Access Control Services (ACS)?

Enforce isolation in multi-endpoint systems.

Q187. What is SR-IOV Initial VFs?

Number of VFs available after boot.

Q188. What is ATS capability ID?

0x000F.

Q189. What is PASID configuration?

Process-level address space routing.

Q190. What is PRI?

Page Request Interface for device-side page faults.

Q191. Why does SR-IOV enumeration fail often?

Incorrect VF BAR size or alignment.

Q192. How many MSI-X vectors can a VF have?

Implementation-dependent.

Q193. What is the SR-IOV System Page Size field?

Supported page sizes for PF/VF.

Q194. What is the Device Control register used for?

Enable:

- Relaxed Ordering

- No Snoop

- Max Payload

- Error reporting

Q195. What is LTR?

Latency Tolerance Reporting.

Q196. What are the possible Completion Status values?

SC, UR, CRS, CA.

Q197. How does OS detect Gen4 capability?

Reads Link Capability 2 field.

Q198. What is secondary bus reset?

Reset issued by root port to downstream devices.

Q199. What is the Bus Master Enable bit?

Allows device to initiate PCIe transactions.

Q200. What is ARI next function number?

In place of traditional function number; supports >8 functions.

🔥 SECTION K — DEBUG & ERROR CASE STUDIES (201–220)

Q201. Why does Gen4 sometimes drop to Gen1 directly?

LTSSM fallback escalates when link is unstable.

Q202. Why does Replay Timer expire immediately?

Transmitter clock faster → ACK arrives late.

Q203. What does a sudden NAK burst indicate?

DFE tap noise or CDR slip.

Q204. Why does LCRC fail only on long packets?

Longer packets → more probability of bit error.

Q205. What causes eye collapse only on lane 3 of x8?

Local crosstalk or defective via.

Q206. Why is SKP OS misalignment common?

Elastic buffer underflow/overflow.

Q207. Why do switches drop NP requests?

NP credits exhausted.

Q208. Why does ATS TLP stall?

Translation cache full.

Q209. Why do VFs get unexpected UR?

Incorrect BDF/routing.

Q210. Why does MRd complete with CA?

Completer lost data → returned CA.

Q211. Why does link flap between L0 and Recovery?

Thermal drift → CDR lock unstable.

Q212. Why does Completion Timeout trigger late?

Software increased timeout limit.

Q213. Why does poison TLP propagate?

Upstream port must forward poison.

Q214. Why does performance drop during SR-IOV?

VC arbitration congestion.

Q215. Why does equalization hang at Phase-1?

Link partner fails to detect TS1/TS2.

Q216. What causes unexpected 4KB violation?

Misaligned payload fragmentation.

Q217. Why does UpdateFC never received?

DLLP lost due to PHY noise.

Q218. Why do Completion tags mismatch?

Requester tag manager bug.

Q219. Why does retry counter overflow?

Excessive NAKs → continuous replay.

Q220. Why does MSI interrupt never come?

MSI enable bit not set in Device Control register.

Got it — here is PART 2 of the 500-question PCIe Gen4 + Company-Specific + UVM Coding Interview Book.

⭐ PCIe GEN4 INTERVIEW Q&A – PART 2 (Questions 51–100)

(All answers included, crisp + spec-accurate)

🔷 51. What is the purpose of SKP ordered sets?

Answer:

- Used to correct clock tolerance compensation between Tx and Rx.

- Negotiated frequency mismatch is absorbed using SKP insertion/removal.

- Must not appear inside TLP/DLLP payloads (only between packets).

🔷 52. When are SKPs inserted in Gen4?

Answer:
Every ~370–400 ns depending on link frequency tolerance.
Higher the Gen rate → more frequent SKP insertion.

🔷 53. What are Electrical Idle Exit Ordered Sets (EIEOS)?

Answer:
Used for electrical idle exit detection and equalization alignment during low-power states (L1, L2).
Ensures Rx correctly detects reactivation of differential pair.

🔷 54. Define Gen4 equalization Phase 0–3 in brief.

Answer:

- Phase 0: Detect that equalization required.

- Phase 1: Tx presents presets; Rx evaluates.

- Phase 2: Rx requests coefficient adjustments.

- Phase 3: Validation and final lock.

🔷 55. What is the role of the Link Control 3 register?

Answer:
Introduced in Gen4 for:

- Equalization enable

- Scaled Flow Control capability

- Lane margining control

- Reduced latency L1 features

🔷 56. What is the Lane Margining mechanism?

Answer:
Allows system software to test receiver eye margin:

- Voltage margin

- Timing margin
Used for predictability and reliability of Gen4/Gen5 links.

🔷 57. Explain Transaction ID (6-bit vs 8-bit vs 10-bit Tag).

Answer:

- Up to Gen2: 5-bit tag

- Gen3/4/5: 8-bit tag

- 10-bit Tag: Optional, supports up to 768 outstanding NP requests
Used by completer to map completions to requests.

🔷 58. What is the Replay Timer?

Answer:
Timers maintained by Tx for ACK/NAK from Rx.
If timer expires → retransmit all packets from replay buffer.

🔷 59. What happens if Rx repeatedly NAKs a TLP?

Answer:
Replay continues until:

- Max Replay Attempts exceeded → Link reset
OR

- Recovery state is entered → retrain link.

🔷 60. Does Gen4 change LCRC polynomial?

Answer:
No — CRC polynomial same as Gen1–Gen5.
PHY layer encoding changed from 128b/130b (Gen3) to improved FEC for Gen4.

🔷 61. How many DLLPs can appear in a sequence number window?

Answer:
Window allows ACK, NAK, InitFC1/2, PM DLLPs, Flow Control, etc.
But sequence number applies only to TLPs, not DLLPs.

🔷 62. What is the Scaled Flow Control (SFC) feature?

Answer:
Reduces cost of credit updates:

- Normal FC uses per-packet credits.

- SFC uses scaling factors.
Introduced in Gen4 to reduce FC DLLP overhead.

🔷 63. Mandatory components for a PCIe VIP?

Answer:

- Phy layer BFM

- Link layer model (ACK/NAK, Replay)

- Transaction layer (TLP formatter)

- Flow control

- Configuration space model

- LTSSM model

- Error injector

- Scoreboard + coverage

- Monitors

- Protocol checkers

🔷 64. What is the difference between MRd and MRd-Lk?

Answer:

- MRd: Normal read request.

- MRd-Lk: Read-return locks, prevents others from modifying data until completion.

🔷 65. Can posted TLPs be retried?

Answer:
No. Only NP TLPs and Cpl packets participate in replay.

🔷 66. What is a poisoned TLP?

Answer:
TLP with EP bit = 1 to indicate data corruption.
Completer must:

- Not treat as fatal

- But mark in logs

- Report AER if enabled

🔷 67. What is Unsupported Request (UR)?

Answer:
Request cannot be processed due to:

- Address not decoded

- Type not supported

- BAR disabled
Completer sends UR Completion.

🔷 68. What is Completer Abort (CA)?

Answer:
Completer intends to service request but internal failure prevented completion.

🔷 69. What is CRS?

Answer:
Configuration Retry Status:
Used when device not ready during enumeration.
Returns CRS completion instead of data.

🔷 70. When does a device return CRS vs UR?

Answer:

- CRS: Device is still initializing.

- UR: Unsupported or invalid config access.

🔷 71. What is the purpose of Vendor-Specific Extended Capabilities (VSEC)?

Answer:
Allow vendors to implement custom control/status registers beyond PCI-SIG spec.

🔷 72. Max_Payload_Size affects which components?

Answer:

- TLP Data field

- Replay buffer depth

- LCRC coverage

- Credit accounting

- Ordering rules (split across 4KB boundary)

🔷 73. Explain 4KB boundary rule for Memory Writes.

Answer:
A single TLP must not cross a 4KB boundary.
Device must split into multiple MWr TLPs.

🔷 74. What is an AtomicOp?

Answer:
Operations like:

- FetchAdd

- Swap

- CAS
Requires both Completer and Requester support.

🔷 75. RX ECRC policy?

Answer:
If ECRC enable bit set →

- Valid ECRC is verified

- Error → log + drop + optional AER

🔷 76. Can LCRC be disabled?

Answer:
No. LCRC is mandatory for link reliability.

🔷 77. What is the difference between RC, EP, and Switch?

Answer:

- RC: Root Complex initiates all traffic.

- EP: Endpoints are targets of requests.

- SW: Routes TLPs based on ID routing.

🔷 78. What is the role of VC0/VC1?

Answer:
Virtual Channels for prioritizing traffic.
VC arbitration decides which TLP transmits.

🔷 79. What is the purpose of TLP Digest?

Answer:
For ECRC – ensures end-to-end data integrity.

🔷 80. Does PCIe allow broadcast?

Answer:
No, only point-to-point.
Multi-target achieved via Root Complex routing.

🔷 81. Why does PCIe use credit-based FC instead of ready-valid?

Answer:
Provides guaranteed buffer availability → no backpressure on wire.

🔷 82. What is the minimum number of lanes for Active State?

Answer:
At least 1 lane must be active for a link.

🔷 83. How does lane polarity inversion work?

Answer:
Receiver automatically detects and swaps polarity (P/N) per lane.

🔷 84. When does lane deskew occur?

Answer:
During Recovery.RcvrLock substate.
Ensures all lanes align correctly.

🔷 85. What TLP types support relaxed ordering?

Answer:
MWr, MRd, AtomicOps, Cpl (optional).
Controlled by RO bit in header.

🔷 86. Can a device send Cpl first before CplD?

Answer:
Yes — completion without data followed by data.
Used for large responses.

🔷 87. When is Configuration Write allowed?

Answer:
Only when device is enabled (Command register bits).

🔷 88. What is PASID used for?

Answer:
Identifies address space for virtualization / IOMMU.

🔷 89. Max PASID size?

Answer:
Either 20 bits or 24 bits depending on implementation.

🔷 90. ATS (Address Translation Service) purpose?

Answer:
Allows device to cache virtual-to-physical translations from IOMMU.

🔷 91. What is PRI?

Answer:
Page Request Interface.
Device requests page faults to OS for DMA operations.

🔷 92. SR-IOV BAR behavior?

Answer:
PF provides VF BARs; VFs inherit capabilities from PF.

🔷 93. What is a Completion Timeout?

Answer:
Requester expects completion within a programmed timeout.
If expired → error logged.

🔷 94. Which packets are ordered strictly?

Answer:

- Config read/write

- FLR

- MRd-Lk

- AtomicOps

🔷 95. What is VDM (Vendor Defined Message)?

Answer:
A message TLP used for vendor-specific communication.

🔷 96. What is the width of device/port number fields?

Answer:

- Bus: 8 bits

- Device: 5 bits

- Function: 3 bits

🔷 97. What is the purpose of Lane Reversal?

Answer:
Allows PCB routing flexibility by reversing lane order (Ln0↔Ln15).

🔷 98. What is MPSS vs MRRS?

Answer:

- MPSS: Max Payload Size supported

- MRRS: Max Read Request Size

🔷 99. In which state does Link Number get assigned?

Answer:
During L0 after configuration.

🔷 100. What are the valid DLLPs?

Answer:

- Ack / Nak

- InitFC1 / InitFC2

- Flow Control Update (normal)

- PM Enter / Exit

- Vendor-defined DLLPs

Great — here is PART 3 of the 500-question PCIe Gen4 + Company-Specific + UVM Coding Interview Book.

⭐ PCIe GEN4 INTERVIEW Q&A – PART 3 (Questions 101–150)

(All answers included, spec-accurate, VIP-oriented)

🔷 101. What is a Completion Boundary?

Answer:
Completion data must be aligned on DWORD (4-byte) boundaries.
A completion may span multiple TLPs but cannot misalign the first DWORD.

🔷 102. When does a completer return multiple CplDs?

Answer:
When returned data exceeds MRRS or MPSS.
Each CplD maintains same Tag but different byte counts.

🔷 103. Can NP requests be merged?

Answer:
No. NP requests always remain independent transactions.

🔷 104. Can posted writes be merged?

Answer:
Yes — hardware can coalesce sequential posted writes to improve bandwidth.

🔷 105. What is Completion ID?

Answer:
Contains Bus/Device/Function of the completer.

🔷 106. Max number of outstanding completions per Tag?

Answer:
One.
Each Tag maps to exactly one in-flight NP request.

🔷 107. Definition of End-to-End CRC (ECRC).

Answer:
CRC computed by requester and verified by completer for entire TLP (including header).

🔷 108. Can LCRC protect DLLP header?

Answer:
Yes — LCRC covers the DLLP payload and type.
DLLP has own CRC separate from TLP LCRC.

🔷 109. What is Fast Training Sequence (FTS)?

Answer:
Special symbols used during equalization, Recovery state transitions, and speed change.

🔷 110. FTS requirement for Gen4 speed change?

Answer:
Transmitter must send at least 16 FTS before entering Recovery.Eq.

🔷 111. What is “Lane equalization successful” condition?

Answer:

- Rx reports coefficients acceptable

- Link transitions to Recovery.Idle

- Then enters L0

🔷 112. What is posted credit starvation?

Answer:
Posted TLPs cannot be transmitted if FC credits for posted headers/data are zero.

🔷 113. What happens if NP credits exhausted?

Answer:
No new NP TLPs (MRd, MRd-Lk, etc.) allowed until FC DLLP updates.

🔷 114. What happens when replay buffer overflows?

Answer:
Link enters Recovery state.
Replay buffer must store entire outstanding window.

🔷 115. Is tag (TLP Tag) reused after wrap-around?

Answer:
Yes — once a tag completes, it becomes available for reuse.

🔷 116. Can DLLPs be retransmitted due to NAK?

Answer:
No. DLLPs are not replayed. Only TLPs are replayed.

🔷 117. Which TLP requires locked ordering?

Answer:
MRd-Lk (Read Lock).

🔷 118. Is PCIe link always symmetrical?

Answer:
No — Tx/Rx lanes may differ in training and equalization states.

🔷 119. What is a prefix TLP?

Answer:
Extra header flits used for features like PASID, PRI, ATS.

🔷 120. Can MRd and MWr have prefix headers?

Answer:
Yes — only if extensions like PASID/ATS are enabled.

🔷 121. How many extended capability headers allowed?

Answer:
Up to 4096 bytes of extended capability space.

🔷 122. What is AER?

Answer:
Advanced Error Reporting mechanism for logging both correctable & uncorrectable errors.

🔷 123. What is Surprise Down error?

Answer:
Occurs when device removed without prior Data Link Layer shutdown.

🔷 124. What is Hot Reset?

Answer:
In-band reset using TLP/DLLP mechanisms.

🔷 125. Warm Reset vs Cold Reset?

Answer:

- Cold reset: full platform reset.

- Warm reset: limited logic reset (LTSSM restart).

- Hot reset: protocol-level reset.

🔷 126. What is PME Turn Off message?

Answer:
Message from OS → device instructing transition to D3hot or D3cold.

🔷 127. What is DPC (Downstream Port Containment)?

Answer:
Contains fatal errors in downstream hierarchy and resets endpoints safely.

🔷 128. What is a Replay Num?

Answer:
Sequence number for TLPs (wraps around at 2048).

🔷 129. Ack/NAK applies to which packets?

Answer:
Only TLPs, not DLLPs.

🔷 130. How is credit scaling factor conveyed?

Answer:
Via Link Control 3 Register for Scaled Flow Control.

🔷 131. Can LCRC error force link retrain?

Answer:
Yes — multiple LCRC errors → trigger link recovery.

🔷 132. What is an Unsupported Request for Completion?

Answer:
Completer rejected NP request → sends Cpl UR.

🔷 133. What is Base Address Register (BAR)?

Answer:
Registers defining memory-mapped or IO-mapped resource windows for endpoints.

🔷 134. How do you determine BAR size?

Answer:
Software writes all 1’s, reads back mask, computes window size.

🔷 135. What is a Prefetchable BAR?

Answer:
BAR whose content can be prefetched by CPUs without side effects.

🔷 136. Can BAR decode be disabled?

Answer:
Yes — via Command Register (Memory Space Enable bit).

🔷 137. What is the Negotiated Link Width?

Answer:
Actual operating lane count after LTSSM transitions.

🔷 138. What is the target of Functions per Device?

Answer:
Max 8 functions per device.

🔷 139. What is ARI (Alternative Routing ID)?

Answer:
Allows >8 functions per device by extending function number to 256.

🔷 140. What is Transaction Layer Packet Prefix (TPH)?

Answer:
Allows tagging of TLPs for software-defined steering.

🔷 141. What is Completion Timeout Disable bit?

Answer:
Disables CTD error to support legacy devices.

🔷 142. What are the three PCIe addressing models?

Answer:

- Memory Space

- IO Space

- Configuration Space

🔷 143. Can PCIe route IO writes?

Answer:
Yes, but increasingly deprecated.

🔷 144. What is the purpose of Secondary Bus Reset?

Answer:
Resets downstream hierarchy of a bridge.

🔷 145. What is the PME_TO_Ack message?

Answer:
Completer acknowledges PME request.

🔷 146. Describe L1 Substates (L1.1 / L1.2).

Answer:

- L1.1: Medium power savings; exit latency moderate

- L1.2: Deep power savings; exit latency high, requires EIEOS

🔷 147. What is “Beacon” in PCIe?

Answer:
Sideband signal used in certain mobile platforms for wake-up.

🔷 148. What is a BAR aperture?

Answer:
The memory range described by BAR.

🔷 149. What is the Vendor ID and Device ID width?

Answer:

- Vendor ID: 16 bits

- Device ID: 16 bits

🔷 150. What is Link Equalization Request (LER)?

Answer:
Indicates that link partner requests additional equalization steps.

Here is PART-4 of the SystemVerilog / Verification Interview Question–Answer list (continuation of earlier parts).

✅ PART-4 — SystemVerilog & UVM Q&A

Q1. What is the difference between rand and randc?

Answer

| Feature | rand | randc |
| --- | --- | --- |
| Pattern | Random every time | Cyclic random (no repeat until full cycle completes) |
| Use case | General random | Coverage-driven sequences where repetition must be avoided |
| Example | rand int a; | randc int b; |

Q2. What is the use of pre_randomize() and post_randomize()?

Answer

| Function | When it Runs | Purpose |
| --- | --- | --- |
| pre_randomize() | Before randomization | Initialize variables, modify constraints dynamically |
| post_randomize() | After randomization | Compute derived values, counters, display, arrays manipulation |

Q3. What is a virtual interface and why do we use it?

Answer

- SystemVerilog classes cannot directly connect to DUT signals.

- So we use virtual interface to pass interfaces into classes.

Used in:

- Drivers

- Monitors

- Scoreboards

Q4. What is the difference between mailbox and queue?

Answer

Mailbox

- Inter-process communication

- Blocking & non-blocking

- Thread-safe

Queue

- Dynamic FIFO container

- No thread safety

- No inter-process sync

Q5. What is the difference between event and semaphore?

Answer

event

- Just triggers a synchronization point.

- No count.

semaphore

- Resource control

- Contains a bucket with resource count

- Used to limit parallel operations

Q6. What is shallow copy vs deep copy?

Shallow copy

- Copies only object handle.

- Both point to same memory.

Deep copy

- Copies entire object contents.

Q7. What is the difference between class and struct?

| Feature | class | struct |
| --- | --- | --- |
| Default | reference type | value type |
| Memory | heap | stack |
| Supports inheritance | Yes | No |
| Used in UVM | Yes | No |

Q8. What are the types of coverage in SV/UVM?

- Code coverage

  - Line, branch, expression, FSM, toggle

- Functional coverage

  - covergroup

  - coverpoint

  - cross coverage

Q9. What is a transactor?

Answer

A transactor converts:

- high-level transaction → pin-level signals
(driver)

- pin-level signals → high-level transaction
(monitor)

Used in all UVM TBs.

Q10. What is objection mechanism?

Answer

Objections are used by components to tell UVM:

- Start: simulation should not end yet

- Drop: simulation can end when all objections are dropped

Q11. What is analysis port?

Answer

Unidirectional broadcasting port used for:

- monitor → scoreboard

- monitor → coverage collector

Many subscribers can listen.

Q12. What is UVM factory?

Answer

Used to:

- create objects dynamically

- override components without changing TB code

Q13. What is a sequence vs sequence item?

Sequence item

- Single transaction

- Fields + constraints

Sequence

- Set of items arranged with timing

- Controls driver through sequencer

**Q14. What is the difference between:

start_item() / finish_item() vs seq_item_port.get_next_item() / item_done() ?

Driver-side handshake

- get_next_item()

- item_done()

Sequence-side handshake

- start_item()

- finish_item()

Q15. What is DPI?

Answer

DPI = Direct Programming Interface
Used to call:

- C → SystemVerilog

- SystemVerilog → C

If you want PART-5, just tell me — “part5”

Here are company-specific interview questions commonly asked in NVIDIA, Intel, Synopsys, AMD, Qualcomm for Design & Verification (SV/UVM + PCIe + AXI) roles — with answers.

I’ll provide:

- Part-1: NVIDIA

- Part-2: Intel

- Part-3: Synopsys

Tell me anytime → “next part” to continue.

🚀 PART-1 — NVIDIA Interview Questions (with answers)

NVIDIA focuses heavily on:

- Complex debugging

- Performance verification

- Cache/Memory subsystem

- PCIe/AXI protocol correctness

- System-level scenario understanding

- UVM deep knowledge

NVIDIA Q1. Explain scoreboard architecture used in your previous project

Answer

NVIDIA expects:

- layered scoreboard

- multiple reference models

- pipe-stages handled

- out-of-order support

Expected reply:

My scoreboard was split into:

- Predictor / Reference Model

- Input FIFO for expected packets

- Output FIFO for actual packets

- Compare module supporting out-of-order matching based on tags/IDs

- Error logger + coverage hooks

NVIDIA Q2. How do you debug a performance bottleneck?

Answer

Steps:

- Add time-stamps at all pipeline entry/exit points.

- Check queue depth behavior.

- Monitor credit usage (PCIe FC, AXI outstanding).

- Verify arbitration fairness.

- Identify back-pressure source → producer or consumer.

NVIDIA Q3. Explain deadlock vs livelock in interconnect

Expected answer

- Deadlock: no forward progress.

- Livelock: always making progress but not useful work (retry, re-arbitrate).

- Happens due to:

  - circular dependency

  - AXI outstanding reads blocked by writes

  - PCIe credit starvation

NVIDIA Q4. What is ILU / LSU / TLB?

Common NVIDIA questions.

Short answers:

- ILU – Integer Logic Unit

- LSU – Load Store Unit

- TLB – Translation Lookaside Buffer (virtual→physical map)

NVIDIA Q5. AXI burst crossing 4KB rule – why?

Because:

- slaves decode upper address bits

- crossing 4KB changes the region

- violates atomicity of a burst

NVIDIA Q6. PCIe replay mechanism

Expect:

- LCRC

- sequence numbers

- ACK/NAK DLLPs

- replay timer

- replay buffer sliding window

NVIDIA Q7. Give 3 real bugs you found

They always ask this.

Structure your answers:

- Bug type

- Root cause

- How you found it

- Fix

🔥 PART-2 — Intel Interview Questions (with answers)

Intel focuses on:

- Computer architecture

- Cache coherence

- Memory ordering

- RTL understanding

- SV/UVM depth

Intel Q1. MESI protocol – explain each state

M – Modified
E – Exclusive
S – Shared
I – Invalid

Expect cross-question:

- What happens during read miss?

- Write miss?

- Eviction?

Intel Q2. What is out-of-order execution?

You must discuss:

- reservation stations

- reorder buffer

- issue queue

- commit stage

Intel Q3. Explain SystemVerilog event regions

Intel loves this.

You must write the sequence:

- Preponed

- Active

- Inactive

- NBA

- Monitor

- Postponed

Intel Q4. How do you generate corner cases in UVM?

Answers expected:

- layered constraints

- constraint random override

- inline constraints

- random-stability checks

- coverage-directed test generation

Intel Q5. What is memory-mapped I/O?

Answer:

Registers of a device are mapped to physical memory space. CPU reads/writes using load-store instructions.

Intel Q6. Why do we prefer functional coverage over code coverage?

Expected answer:

Code coverage checks implementation, not specification.
Functional coverage ensures all user-visible features are tested.

Intel Q7. What is a synchronizer? Why 2-flop synchronizer?

To avoid metastability when crossing clock domains.

🏆 PART-3 — Synopsys Interview Questions (with answers)

Synopsys focuses on:

- UVM methodology depth

- Testbench architecture

- Constraint solving

- Coverage optimization

- Linting / CDC / SVA

- Protocol-heavy IP block (PCIe, USB, DDR)

Synopsys Q1. Explain factory override with example

They want:

- type override

- instance override

- use of set_type_override_by_type

- use of set_inst_override_by_name

Synopsys Q2. Why UVM uses TLM ports instead of direct calling?

Expected answer:

To avoid tight coupling, enable multiple subscribers, and support factory overrides.

Synopsys Q3. Difference between UVM config_db and RAL models

- config_db: passes configuration

- RAL: register level abstraction for SW model testing

Synopsys Q4. What is a virtual sequence? When do you use it?

Used for:

- multi-agent coordination

- SoC-level tests

- parallel traffic generation

Synopsys Q5. UVM phases in order

You MUST know:

build_phase

connect_phase

end_of_elaboration_phase

start_of_simulation_phase

run_phase

extract_phase

check_phase

report_phase

Synopsys Q6. What is constraint randomization limit?

Synopsys tool interviews expect:

Randomization complexity increases exponentially, solver backtracks multiple times.
Use solve … before, distribution constraints, and custom solve order to optimize.

**Synopsys Q7. Write a simple SVA:

When req goes high, ack must come within 3 cycles**

property p_req_ack;

@(posedge clk) req |=> ##[1:3] ack;

endproperty

assert property (p_req_ack);

Below is PART-4 — more company-specific PCIe + UVM + SoC + AXI interview questions from:

✔ Qualcomm

✔ AMD

✔ Broadcom

✔ Cadence

✔ Extra Deep NVIDIA/Intel questions

✔ PCIe Gen4/Gen5 corner questions

✔ UVM coding tasks

All with answers.

🔥 PART-4 — Qualcomm, AMD, Broadcom, Cadence, Extra NVIDIA/Intel Advanced Questions

🟥 QUALCOMM INTERVIEW QUESTIONS (with answers)

Qualcomm focuses on:

- PCIe Gen4/Gen5

- Interconnect (NoC / AXI / CHI)

- Low-power

- Clocking + CDC

- Debug analysis

Q1. What is LTR (Latency Tolerance Reporting)? Why is it used?

Answer

LTR is used by devices to tell the Root Complex how long they can tolerate delay before performance drops.

Used for:

- power saving

- turning off unused clock domains

- dynamic link power management

LTR fields:

- SNOOP latency

- NON-SNOOP latency

Q2. How does a NoC (Network-on-Chip) differ from AXI crossbar?

AXI crossbar → point-to-point predictable
NoC → packetized, arbitration based, supports:

- routing

- QoS

- virtual channels

- congestion feedback

Q3. PCIe: What is Optimized Buffer Flush? (QCOM specific)

Some Qualcomm IPs implement proprietary features where:

- Cpl buffers flush before link low-power entry

- Reduces replay chances after L1 exit

Q4. How do you verify low-power transitions (L1 / L1.1 / L1.2)?

Answer must include:

- Toggle PM_Enter_L1 and PM_Enter_L1.2

- Check:

  - EIEOS pattern

  - TS1/TS2 exchanges

  - LTSSM sub-states

- Exit sequence checking

- Ensure TLP blocked during exit

Q5. AXI interleaving – give example of legal & illegal cases

Legal:

- ID = 5: Burst A, Burst B interleave
Illegal:

- Writing to same address region with interleaving

- Changing transaction ID mid-burst

Q6. Write SVA to verify that once L1 is requested, no TLP is transmitted

property no_tlp_during_l1;

pm_enter_l1 |-> !tlp_tx until pm_exit_l1;

endproperty

assert property(no_tlp_during_l1);

🟩 AMD INTERVIEW QUESTIONS (with answers)

AMD focuses on:

- GPU + PCIe

- Multi-function devices

- SR-IOV

- Virtualization

- High bandwidth links

Q1. Explain ATS (Address Translation Service)

Used by devices to request address translation from IOMMU.

ATS TLP sequence:

- ATS Translation Request

- ATS Translation Completion

- Device caches translation

- Must invalidate on IOMMU notification

Q2. What is PASID?

Process Address Space ID.
Used in:

- Shared/interleaved address spaces

- Virtual machines

- GPU compute queues

Q3. Why do GPUs require multiple tags/credits?

For massive parallelism:

- thousands of concurrent loads/stores

- GPU → uses hundreds of outstanding TLPs
So:

- 10-bit Tag support mandatory

- Scaled Flow Control required

Q4. What is SR-IOV? Explain with example.

SR-IOV = Single Root I/O Virtualization.
PF (physical function) → owns HW
VF (virtual function) → exposed to guest OS
Scenario:

- one NIC

- 64 VFs

- each VM sees its own virtual NIC

Q5. Explain PCIe Relaxed Ordering and No-Snoop use in GPUs

GPUs often:

- enable Relaxed ordering for parallel loads

- use No-Snoop for cache-bypass transfers

Q6. AMD likes this coding question:

Write a UVM sequence sending 50 Memory Writes with unique tags (Gen4, 10-bit)

class memwr_seq extends uvm_sequence #(tlp_item);

rand bit [9:0] used_tags[$];

task body();

repeat (50) begin

tlp_item req = tlp_item::type_id::create("req");

assert(req.randomize() with {

fmt_type inside {MEM_WR32, MEM_WR64};

tag inside {[0:1023]};

!(tag inside used_tags);

});

used_tags.push_back(req.tag);

start_item(req);

finish_item(req);

end

endtask

endclass

🟦 BROADCOM INTERVIEW QUESTIONS (with answers)

Broadcom focuses on:

- PCIe switches

- Multi-port RC

- Downstream Port logic

- Completion routing

- Virtual channels

Q1. Explain completion routing rules in PCIe switch

Switch must route based on:

- Requester ID

- Tag

- Function number

Switch routing for Cpl:

- Check Cpl's Completion ID

- Lookup forwarding table

- Route to the correct upstream/downstream port

Q2. What is CRS (Configuration Retry Status)?

When endpoint not ready → replies with CRS Completion.
Root Complex must:

- Retry until CRS clears

- Software reads real value later

Q3. Explain Virtual Channel arbitration in switch

VC0 → Best effort
VC1-7 → High priority traffic
Switch uses:

- round-robin

- weighted round robin

- strict priority
based on QoS

Q4. What is ACS? (Access Control Services)

Ensures:

- no peer-to-peer access unless allowed

- blocks spoofing

- needed for SR-IOV

ACS controls:

- P2P requests

- Upstream forwarding

- Translation blocking

Q5. Broadcom loves this: What are Ordering Rules for NP vs P TLPs?

You MUST answer:

- NP must not bypass NP ahead

- NP may bypass P

- Cpl must not bypass Cpl to same requester

- Rules relax with "Relaxed Ordering" bit

🟪 CADENCE INTERVIEW QUESTIONS (with answers)

Cadence focuses on:

- VIP architecture

- BFMs

- RAL

- SVA

- Multi-protocol VIP integration

Q1. Explain complete VIP architecture (agent level)

Include:

- Sequencer

- Driver

- Monitor

- Coverage collector

- Scoreboard

- Subscriber

- Config object

- Virtual sequencer

Q2. VIP Error Injection Techniques

Cadence expects:

- LCRC corruption

- 4KB crossing violation

- Malformed TLP

- Replay timer manipulation

- DLLP credit corruption

Q3. What is Passive Agent? Why used?

Monitors traffic → used in SoC for:

- non-intrusive checking

- coverage

- protocol compliance

Q4. What is a Predictor? When do you use it?

Predictor builds expected packets for scoreboard.
Used when:

- DUT transforms request to completion

- Need reference model behavior

Q5. How do you test link bring-up?

Test through:

- LTSSM sequences

- TS1/TS2 exchange

- Link number validation

- L0 entry

- Inject training errors

- Replay buffer behavior

🧡 EXTRA ADVANCED NVIDIA / INTEL QUESTIONS

These are asked for Senior DV roles.

Q1. Explain PCIe posted/NP/Cpl ordering with examples

Expected:

- P never requires Cpl

- NP requires Cpl

- Cpl must not pass NP to same requester

- Relaxed ordering rules

Q2. How does reply combining work in completion buffer?

Answer:

- combine completions with same Tag

- merge data payload

- reorder if segments arrive fragmented

- check Byte Count and Lower Address

Q3. Replay timer expiry – what happens?

- Replay timeout

- Send NAK

- Retransmit from replay buffer

- Increment replay counter

- If multiple failures → Link Down

Q4. How does Electrical Idle Exit get detected?

By RX:

- CDR locks

- Detect Redriver activity

- TS1/TS2 observation

- Skip ordered sets

Q5. Why does Intel require strict MISC FIFO clean-up logic?

Because:

- OoO execution

- reorder windows

- requires speculative prefetch safe rollback

🎯 UVM CODING QUESTIONS (ADVANCED)

Q1. Write constraint so Memory Read 64 always aligned to DW

constraint align_dw {

addr % 4 == 0;

}

Q2. Write a coverage model for Tag, type, length

covergroup tlp_cg @(posedge clk);

cp_tag   : coverpoint tag   { bins all[] = {[0:1023]}; }

cp_type  : coverpoint type  { bins mem = {MEM_RD32, MEM_WR32}; }

cp_len   : coverpoint length { bins len[] = {[1:32]}; }

endgroup

Q3. Create UVM scoreboard skeleton for Req→Cpl matching

class pcie_scoreboard extends uvm_scoreboard;

`uvm_component_utils(pcie_scoreboard)

mailbox #(tlp) exp_mbx, act_mbx;

function new(string name="pcie_scoreboard", uvm_component parent);

super.new(name,parent);

exp_mbx = new();

act_mbx = new();

endfunction

task run_phase(uvm_phase phase);

tlp exp, act;

forever begin

exp_mbx.get(exp);

act_mbx.get(act);

if(exp.tag != act.tag)

`uvm_error("SB", "Tag mismatch")

end

endtask

endclass

Q4. Write a UVM monitor for extracting TLP header

task monitor();

forever begin

@(posedge vif.rx_valid);

tlp_item t = new();

t.fmt  = vif.rx_fmt;

t.type = vif.rx_type;

t.tag  = vif.rx_tag;

port.write(t);

end

endtask

Q5. Write SVA for credit never going negative

assert property (@(posedge clk) fc_credit >= 0);

Here is PART-5 — continuing PCIe Gen4/Gen5, UVM coding, and company-specific advanced questions.

This part focuses on UVM scenarios, constraint solving, edge-case PCIe behavior, VIP implementation, and interview-style problem solving.

🔹 PART-5 — Advanced PCIe + UVM + VIP Coding Qs (201–250)

PCIe Protocol & Edge-Case Questions

Q201. Explain Completion Timeout for multi-function devices

Answer:

- Each function has separate timeout register.

- If completion not received within programmed CT limit → error logged.

- Multi-function devices must monitor all outstanding tags per function.

Q202. What happens if MRd crosses 4KB boundary?

Answer:

- Device must split into two TLPs.

- Each TLP adheres to MPSS (Max Payload Size).

- Replay buffer tracks both TLPs separately.

Q203. How is Replay Buffer size calculated?

Answer:

- Max outstanding NP requests × Max Payload Size / PCIe lane bandwidth.

- Must accommodate all in-flight requests + optional retries.

Q204. What is ECRC vs LCRC?

| Feature | LCRC | ECRC |
| --- | --- | --- |
| Scope | TLP only | End-to-end (Requester→Completer) |
| Mandatory | Yes | Optional (enable per device) |
| Detects | Transmission errors | Data corruption |

Q205. Explain Electrical Idle Exit with diagrams

Answer:

- TX sends TS1/TS2 / SKP ordered sets

- RX detects CDR lock

- LTSSM transitions from Recovery → L0

- TLP flow resumes

(Diagram: TS1/TS2 → Recovery.RcvrLock → L0)

Q206. Gen4 Equalization Phase diagrams

Answer:

- Phase 0: Detect

- Phase 1: Tx preset coefficients

- Phase 2: Rx feedback (request adjustments)

- Phase 3: Verification → L0

(Diagram: Tx→Rx↔Coefficient feedback loop)

Q207. What is Relaxed Ordering? Example

Answer:

- Allows NP TLPs to pass posted TLPs for performance.

- Example: MRd ID=5, MWr ID=6 → reordered on wire if RO bit set.

Q208. How is AtomicOp verified in VIP?

Answer:

- VIP must generate aligned operand

- Completion matches original request

- Tag and ID consistency

- Handle multi-TLP operation (if >4KB)

Q209. Why do multi-lane links require deskew?

Answer:

- Each lane may have skew due to PCB trace length differences

- Deskew ensures correct byte alignment at RX

Q210. What is Scaled Flow Control?

Answer:

- Reduces FC DLLP traffic overhead

- Credits reported with scaling factor

- Requires calculation at both TX/RX

🔹 UVM Coding & Testbench Questions

Q211. Build a TLP generator sequence with random alignment

class tlp_seq extends uvm_sequence #(tlp_item);

rand bit [3:0] offset;

rand tlp_item item;

constraint align {

offset < 4; // byte alignment

}

task body();

start_item(item);

item.addr = (item.addr & ~4) + offset;

finish_item(item);

endtask

endclass

Q212. Scoreboard for Req→Cpl tracking with 10-bit Tag

class sb extends uvm_scoreboard;

mailbox #(tlp_item) exp_mb, act_mb;

task run_phase(uvm_phase phase);

tlp_item exp, act;

forever begin

exp_mb.get(exp);

act_mb.get(act);

if(exp.tag != act.tag) `uvm_error("SB","Tag mismatch")

end

endtask

endclass

Q213. Coverage model for Tag × Type × Length

covergroup cg @(posedge clk);

coverpoint tag;

coverpoint type;

coverpoint length;

cross tag, type, length;

endgroup

Q214. Create UVM monitor to extract header fields

task monitor();

forever begin

@(posedge vif.rx_valid);

tlp_item t = new();

t.fmt  = vif.rx_fmt;

t.type = vif.rx_type;

t.tag  = vif.rx_tag;

port.write(t);

end

endtask

Q215. Write constraint to ensure 4KB-safe TLP

constraint fourkb_safe {

(addr[11:0] + length*4) <= 12'hFFF;

}

Q216. How to inject ECRC error in VIP

- Modify TLP header or payload CRC

- Mark TLP as poisoned

- Observe scoreboard / monitor catches it

Q217. Generate NP TLP bursts respecting tag limits

- 10-bit tag → max 1024 outstanding NP requests

- Randomize tag avoiding collisions

- Use randc or array of used tags

Q218. How to model BAR sizing in config space

- Write all 1s to BAR

- Read back value → mask

- Compute size = ~mask + 1

- Set BAR register in VIP model

Q219. Multi-agent VIP – how to coordinate sequences

- Use virtual sequencer

- Sequence triggers sub-sequences on multiple agents

- Each agent receives transactions via sequencer

Q220. How to test LTSSM in UVM

- VIP must drive TS1/TS2 ordered sets

- Observe state transitions

- Check Recovery, Equalization, L0 entry

- Scoreboard tracks state vs expected

🔹 Company-Specific Coding / Debug Examples

Synopsys

- Factory override: override driver/monitor classes without TB change

- Use TLM ports for broadcast to scoreboard & coverage

NVIDIA

- GPU SR-IOV: multiple VFs, each with 10-bit tag

- Stress-test completion matching and relaxed ordering

Intel

- ATS + PASID verification

- Memory read/write latency scenarios

- Out-of-order requests with multiple outstanding tags

Broadcom

- Switch completion routing verification

- VC arbitration check with round-robin & weighted RR

- ACS / P2P request check

AMD

- Multi-function devices with independent replay / CT timers

- L1/L1.1 exit latency verification

- Relaxed ordering / non-snoop path coverage

✅ Summary of PART-5

- PCIe edge-case behaviors: 4KB boundary, ECRC, LCRC, replay, equalization

- UVM coding: sequences, monitors, scoreboard, coverage

- Company-specific stress points: NVIDIA/Intel/AMD/Broadcom/Synopsys